W1L1 Embedded Systems Introduction

- What is an embedded system?

  - Any system that includes a programmable computer but is not itself a general-purpose computer.

  - A microprocessor-based system that is built to control a function or range of functions and is not designed to be programmed by the end user.

  - A system designed to perform one specific task and is optimized for that application.

- How do embedded systems differ from pcs?

  - Embedded systems

    - Specific application that is known at design time

    - Not programmable by end user

    - Optimized for fixed runtime requirements

    - Criteria

      - Cost

      - Reliability

      - Power consumption

      - Size

  - PC

    - Broad class of applications

    - Programmable by end user

    - Faster is better

    - Criteria

      - Cost

      - Average speed

- Components of an embedded system

  - Processor

  - Some sort of interrupt

    - Used for errors such as overvoltage.

    - Timer interrupts.

  - Memory

  - I/O

  - Oscillator

  - Bus control

  - Bus control

  - Timers

- Characteristics of Embedded Systems

  - Must be dependable

  - Must be efficient

  - Must meet real time constraints

  - Must meet time to market constraints

- Embedded Systems Technologies

  - Application-Specific Integrated Circuit (ASIC)

  - Application-Specific Standard Parts(ASSP)

  - System-on-chip (SOC)

  - Field Programmable Gate Array (FPGA)

  - System-on-Programmable-Chip (SOPC)

  - SOPC compared to ASIC or ASSP

    - Pros

      - Short lead time - how much time it takes to receive the part.

      - Lower NRE (non-reoccurring engineering)

      - Design changes are easy(reprogram)

    - Cons

      - High power

      - Cost is constant despite volume

      - Must choose from off the shelf parts

        - Not optimized.

- System on Programmable Chip

  - Designing an SOPC

    - Hard Cores: Select a chip with required hard cores (IP).

      - RAM

      - ADC

      - DSP Logic

    - Soft Core: Add required soft cores (IP).

      - Things added by the manufacturer.

      - NIOS II

      - Multipliers and dividers

      - Interfaces (USB, I2C, SPI, etc.)

    - Add user logic to tie everything together.

- Embedded System Design Considerations

  - System Partitioning

    - Determine where each function should be implemented.

  - Design challenges of embedded systems

    - Test benches are as good as they are designed.

- Embedded System Design Flow

  - Test & Verification

    - Critical to successful design.

    - The sooner bugs and errors are detected, the easier and less expensive they are to fix.

# W1L2 SOPC and Cores

- Digital Systems – Brief History

- Microprocessor

  - At a basic level, a microprocessor is a state machine.

  - States: Fetch -> Decode instructions -> Execute Instructions (ALU performs task in another register) -> Save information -> Write back information -> Fetch.

- Moore’s Law

  - Since the invention of the IC in 1958, the number of transistors that can be placed doubles roughly every two years.

- Where are we now?

  - SOPC compared to custom IC

    - Shorter lead time (+)

    - Lower NRE (+)

    - Design changes are easy (reprogram) (+)

    - Higher power (-)

- Tradeoffs

  - ASIC: designing the VHDL.

  - Full custom VLSI design: Placing components.

- System on programmable chips

  - Typically includes

    - Intellectual Property

    - Custom hardware

    - Application-specific hardware components

- IP Cores in SOPC

  - What are IP Cores?

    - Can range in complexity

    - Simple arithmetic operators

    - System-level building blocks

    - Specialized functions

  - What is soft-core processor?

    - A microprocessor fully described in software, usually an HDL, which can be synthesized in programmable hardware, such as FPGA’s.

  - What is a hard processor?

    - The processor is embedded in the silicon of the FPGA.

- Processor Cores (Slide 22) (will be on a quiz)